RK Tek  734/827 Big Bore and 82mm “Drop In” Piston Installation

- Clean ALL new and old parts thoroughly with soapy water to assure no foreign debris is present

- Set piston ring end gap between .016” and .018”.  If larger then leave it alone.

To set end gap:

a) Place 1 ring in cylinder

b) Level ring in cylinder by using the piston as a leveling device. Measure gap with a feeler gauge.

c) Using a wet-stone or something similar, gently grind on one end of the ring until desired

clearance is obtained

d) Repeat process for each ring.

e) MAKE SURE you know which rings are gapped for which cylinders!

- Install rings on pistons

- Install pistons and top end bearing onto connecting rods. If you are using DOO factory cage-less bearings…DO NOT install piston cir-clips on piston until AFTER the piston is on the rod.

- Install wrist pin and cir-clips onto piston. MAKE SURE cir-clips are full seated in groove!

- Install, RKT supplied, .5mm gasket, base gasket. To assure a flat seal. MAKE SURE case and cylinder base is free of ANY grease or foreign debris. USING NO SEALER!! Install MAG cylinder. Hand-tighten bolts. Make sure the gasket has not been displaced during cylinder installation.

- Install PTO cylinder using the same cautions as above.

- Install RAVE gaskets (not supplied) under each RAVE guillotine. Make sure RAVE base housing has exhaust supply hole line up with supply hole on cylinder. Bleed hole toward belly-pan.

- Torque cylinder bolts to 27 ft/lbs and then final torque bolts to 32ft/lbs.

- Install Head as per supplied instructions. Torque @ 22ft/lbs

- Do not run the BR9ECS plug run the BR9EYA plugs (Arctic Cat) or BR9ES gap the plug @.020”

- PREMIX a 150:1 oil ratio in 1st tank of fuel. RKT recommends to always premix at 150:1 in addition to your oil injection system. DO NOT USE AMSOIL!! Break in engine with  mineral based oil for the 1st 100 miles.

- IMPORTANT: DO NOT USE ANY aftermarket silencer on this set-up. Use the stock pipe and muffler. This “kit” was not tested with any other pipe/silencer combination.

- Fire engine and check for base gasket leakage. Make sure there are no air pockets in the coolant system.

- Run engine at varying throttle positions for the 1st 50 miles checking piston wash and plug color (and adjusting accordingly), then ride it like normal! NOTE: The 800 and 827 engine are 8400-8500rpm engines! 734 kit runs at 8000rpm. NOTE: These engines  take 100 miles for the rings to fully seat.

- Jetting (734 kit):> on the Flat-Side carbs, start with 370 mains, 20/17.5 pilots and the needle from the 500SS carbs.. Round Slide:->> 310 mains, 40 pilots, stock needle. NOTE: These are JUST starting recommendations…FINAL jetting could be much different (usually MUCH lower)

- Jetting (827 kit): If you have hooded needles (SOME ‘03 and newer) start with 390 mains and work down from there (probably end up at 350’s) richen needle 1 clip. For ‘01 and ‘02 , non hooded carbs, start with 480 mains and work down.

- Jetting 82mm drop in : If you have hooded needles (04 (Some 03 models) and newer) start with 370 mains and work down from there (probably end up at 350’s) richen needle 1 clip position. For ‘01 and ‘02 , non hooded carbs, start with 490 mains and work down and add extra shim under needle.

800R engine:  450 main jet and raise needle 1 clip. 441 ramps, 12g pin weight Pur/Blu spring

- Clutching: RKT recommends changing the primary spring to the Ski Doo Purple-Purple Spring.

If below 3000ft fully load the pins and set clicker to #2 setting. If above 3000ft-> start with 14g (800HO) of pin weight.  Run the engine ON SNOW under load and adjust pin weight until the engine is pulling 8000-8300rpms (800 and 827)..then click up 1-2 settings on your primary(#3 or #4) and you should be close.   NOTE: do NOT assume that if you are pulling the 8400-8500 rpms that you have the correct LOAD on the engine… ALWAYS load the engine UNTIL it will no longer pull the 8400-8500 rpms then you will be in the correct area for clutching. Do the same for 734 kit but rpm is 8000 vs. 8500.