CR 500 Installation Instructions

## RK Tek

1245 Industrial Park Rd.

Preston, ID 83263

208-852-3221

Email: torqueline@2strokeheads.com

Web:       www.2strokeheads.com

- Drain coolant from engine.

- Disconnect hoses on head.

- Remove original head

- Remove Gasket from cylinder . Make sure cylinder deck is COMPLETELY free of gasket (old) material.

- Install brass barb fittings in head shell. Make sure you use Teflon tape on threads.

- Wrap cylinder stud threads with Teflon tape.

- Install 1 Large o-ring into RKT head shell and 1 large o-ring into the dome’s groove.

- Set dome inside head shell.

- Slide head shell/dome assembly onto cylinder.

- Install, RKT supplied, SMALL stud o-rings around cylinder studs into head shell groove. NOTE: these o-rings go on the OUTSIDE of the head shell under the nuts.

- Install stud nuts .

- Torque nuts at 22 ft/lbs. in a circular pattern.

- Re-connect coolant hoses.

- Secure coolant hoses with hose clamps.

- Fill coolant reservoir with a mix of 50/50 anti-freeze and water.

- Start engine and let idle for 2  minutes in order to circulate coolant.

- ENJOY!!!

Jetting: 	There is no way for RK Tek to know all the possible jetting combinations for every engine. We do recommend jetting up 2 sizes on the main jet as a starting point. NOTE: This is only a starting point. The proper jet could be many sizes higher or lower depending on other modifications on the engine. Monitor piston was to arrive at the correct main jet.