700 Arctic Cat Head/Keyway Installation Instructions

# RK Tek

## 1245 Industrial Park Rd.

## Preston, ID 83263

## 208-852-3221 Fax 208-852-3228

email: torqueline@2strokeheads.com

www.2strokeheads.com

- Drain original coolant from top of engine. This is most easily done by using a shop-vac and a small hose to suck the coolant out from the inlet. Remove spark plugs.

2.	Install the puck sealing O-rings (2) on to each puck (dome) top.

3.	Install pucks into head shell(make sure that the puck O-rings are installed). NOTE: These pucks only go in one way! Line up the inscribed "X" on the puck with the head shell “X”. Make sure that you have the puck level when installing it. Line up and SLOWLY press them into place.  If you can not see the “X” you can visually spot on the dome where one “ear” is different shaped than the other ears. This ear will go to the coolant inlet side of the head shell.

4.	Lightly grease the tops of the O-rings on the cylinders to assure that they stay in place.

5.	Install the completed head assembly onto the engine.

6.	Torque head bolts in the proper pattern first at 18 ft/lbs and then go back and torque them at 22 ft/lbs. Do not over torque bolts!

7.	Pour new coolant back into engine .Pour slowly until full.

8.	Install plugs and start engine and let get hot until thermostat opens.

- MAKE SURE that you have no air lock in the system!!

- The offset keyway (if ordered)  NEEDS to go in reverse of the instructions supplied with the keyway. In otherwords.. This keyway needs to be installed in a manner that will move the flywheel COUNTER-CLOCKWISE… The “offset” portion of the keyway should be set to the LEFT when installing the keyway.